|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5/PACP-12 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-06(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.12)

Agenda Item 1.12

1.12	to protect the primary services in the band 37–38 GHz from interference resulting from aeronautical mobile service operations, taking into account the results of ITU-R studies, in accordance with Resolution 754 (WRC-07)

Introduction

ITU-R studies have concluded that “low power aircraft transmitters such as WAIC will not be able to use the band without causing harmful interference to the SRS earth stations”,  and the pfd spectral density limit included in Method B is highly impracticable, APT Members support the exclusion of AMS from the MS allocation in the 37-38 GHz band.

The related proposals are given below.

Proposals

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD 		ASP /1.12/1

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 37-37.5 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| ... |  |  |

Reason: In the light of the results of ITU-R studies, AMS service would cause harmful interference to the SRS Earth stations and recognizing that AMS would be interfered from FS, restriction of current “MOBILE” allocation in the band 37-38 GHz to “MOBILE except aeronautical mobile” allocation is proposed.

SUP	ASP/1.12/2

resolution  754  (wrc-07)

Consideration of modification of the aeronautical component of
the mobile service allocation in the 37-38 GHz band for
protection of other primary services in the band

Reason: Studies required by Resolution 754 (WRC-07) have been completed.

_______________