|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-15 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-10(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.15)

Agenda Item 1.15

1.15	consider possible allocations in the range 3-50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU-R studies, in accordance with Resolution 612 (WRC-07);

Introduction

APT Members support new allocations to the RLS for oceanographic radar applications with appropriate regulatory provisions for a possible sharing with existing services to enable the most efficient use of the spectrum and mitigate interference to existing services.

APT Members recognize that the combination of allocation on primary and secondary basis is practical on sharing and protecting with various existing services in multiple sub-bands from lower frequencies for supporting long-range operation to higher frequencies for supporting higher resolution in short-range operation.

Therefore, APT Members support Method C in the CPM Report to WRC-12, with a regulatory provision also applied to the secondary allocations to RLS.

The related proposals are given below.

Proposals

Proposals relating to secondary allocation on 3 155-3 200 kHz bands

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD		ASP/1.15/1

2 194-3 230 kHz

3 155-3 200 FIXED MOBILE except aeronautical mobile (R) Radiolocation ADD 5.A115 5.116 5.117

ADD		ASP/1.15/2

5.A115	For the use of the bands 3 155-3 200 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the lower frequencies to the RLS for oceanographic radar applications on long-range operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to secondary allocation on 4 438-4 650 kHz bands

MOD		ASP/1.15/3

3 230-5 003 kHz

| 4 438-4 488 FIXED MOBILE except aeronautical mobile (R) Radiolocation ADD 5.B115 | 4 438-4 488 FIXED MOBILE except aeronautical mobile Radiolocation ADD 5.B115 |
| --- | --- |
| 4 488-4 650 FIXED MOBILE except aeronautical mobile (R) | 4 488-4 650 FIXED MOBILE except aeronautical mobile |

ADD		ASP/1.15/4

5.B115	For the use of the bands 4 438-4 488 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the lower frequencies to the RLS for oceanographic radar applications on long-range operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to primary allocation on 5 060-5 450 kHz bands

MOD		ASP/1.15/5

5 003-7 450 kHz

5 250-5 400 FIXED Mobile except aeronautical mobile

5 400-5 450 FIXED Mobile except aeronautical mobile RADIOLOCATION ADD 5.C115

ADD		ASP/1.15/6

5.C115	For the use of the bands 5 400-5 450 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the lower frequencies to the RLS for oceanographic radar applications on long-range operation with appropriate regulatory provisions.

Proposals relating to secondary allocation on 12 100-12 230 kHz bands

MOD	ASP/1.15/7

7 450-13 360 kHz

12 100-12 200 FIXED Radiolocation ADD 5.D115

12 200-12 230 FIXED

ADD		ASP/1.15/8

5.D115	For the use of the bands 12 100-12 200 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the middle-range frequencies to the RLS for oceanographic radar applications on middle-range operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to secondary allocation on 13 870-14 000 kHz bands

MOD	ASP/1.15/9

13 360-18 030 kHz

13 870-13 900 FIXED Mobile except aeronautical mobile (R)

13 900-14 000 FIXED Mobile except aeronautical mobile (R) Radiolocation ADD 5.E115

ADD		ASP/1.15/10

5.E115	For the use of the bands 13 900-14 000 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the middle-range frequencies to the RLS for oceanographic radar applications on middle-range operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to primary allocation on 22-24 MHz bands

MOD	ASP/1.15/11

18 030-23 350 kHz

22 855-23 000 FIXED RADIOLOCATION ADD 5,F115 5.156

23 380-27 500 kHz

24 000-24 450 FIXED LAND MOBILE

24 450-24 600 FIXED LAND MOBILE RADIOLOCATION ADD 5,F115

24 600-24 890 FIXED LAND MOBILE

ADD		ASP/1.15/12

5.F115	For the use of the bands 22 855-23 000kHz and 24 450-24 600 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the higher frequencies to the RLS for oceanographic radar applications on short -range operation.

Proposals relating to secondary allocation on 26-27 MHz bands

MOD	ASP/1.15/13

18 030-23 350 kHz

26 175-27 325 FIXED MOBILE except aeronautical mobile Radiolocation ADD 5,G115 5.150

26 325-27 350 FIXED MOBILE except aeronautical mobile 5.150

27 350-27 500 FIXED MOBILE except aeronautical mobile Radiolocation ADD 5,G115 5.150

ADD		ASP/1.15/14

5.G115	For the use of the bands 26 175-26 325 kHz and 27 350-27 500 kHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the higher frequencies to the RLS for oceanographic radar applications on short -range operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to secondary allocation on 41-44 MHz bands

MOD	ASP/1.15/15

27.5-47 MHz

41.015-41.515 FIXED MOBILE Radiolocation ADD 5,H115 5.160 5.161

41.515-44 FIXED MOBILE 5.160 5.161

ADD		ASP/1.15/16

5.H115	For the use of the bands 41.015-41.515 MHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the higher frequencies to the RLS for oceanographic radar applications on short -range and high spatial resolution operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to secondary allocation on 44-47 MHz bands

MOD	ASP/1.15/17

27.5-47 MHz

44-46.5 FIXED MOBILE 5.162 5.162A

46.5-47 FIXED MOBILE Radiolocation ADD 5.I115 5.162 5.162A

ADD		ASP/1.15/18

5.I115	For the use of the bands 46.5-47 MHz by the radiolocation service, Resolution 612 (Rev.WRC-12) applies.     (WRC-12)

Reason:  APT Members support new allocations in the higher frequencies to the RLS for oceanographic radar applications on short -range and high spatial resolution operation with high protection against existing and future use of fix and mobile services in targeted bands.

Proposals relating to necessary regulatory provisions

APPENDIX 4  (Rev.WRC-07)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

MOD 		ASP/1.15/19

table 1

Characteristics for terrestrial services

| Column No. | Item identifier | Notice related to Description of data items and requirements | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |  |  |
| 3 |  | CALL SIGN AND STATION IDENTIFICATION |  |  |  |  |  |  |  |  |
| 3.1 | 3A1 | the call sign used in accordance with Article 19 | O | O | + |  |  |  | O | 3A1 |
|  |  | In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (WRC-12)), or standard frequency and time signal service, in the application of Article 11, required if the station identification (3A2) is not provided |  |  |  |  |  |  |  |  |
| 3.2 | 3A2 | the station identification used in accordance with Article 19 | O | O | + |  |  |  | O | 3A2 |
|  |  | In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (WRC-12)), or standard frequency and time signal service, in the application of Article 11, required if the call sign (3A1) is not provided |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |

Reason: APT Members support new allocations to the RLS for oceanographic radar applications with the station identification.

ARTICLE  19

Identification of stations

Section I  –  General provisions

MOD	ASP/1.15/20

19.1.1	In the present state of the technique, it is recognized nevertheless that the transmission of identifying signals for certain radio systems (e.g. radiodetermination, radio relay systems and space systems) is not always possible. However, for stations in the radiolocation service, notified to the Bureau or brought into use after 15 February 2012, in bands between 3 and 50 MHz (operating in accordance with Resolution 612 (WRC-12)), the provisions of No. 19.51 shall be applied.

Reason: APT Members support new allocations to the RLS for oceanographic radar applications with the station identification.

MOD	ASP/1.15/21

RESOLUTION 612 (Rev.WRC-12)

Use of the radiolocation service between 3 and 50 MHz to
support oceanographic radar operations

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that there is increasing interest, on a global basis, in the operation of oceanographic radars for measurement of coastal sea surface conditions to support environmental, oceanographic, meteorological, climatological, maritime and disaster mitigation operations;

b)	that oceanographic radars are also known in parts of the world as HF ocean radars, HF wave height sensing radars or HF surface wave radars;

c)	that oceanographic radars operate through the use of ground-wave propagation;

d)	that oceanographic radar technology has applications in global maritime domain awareness by allowing the long-range sensing of surface vessels, which provides a benefit to the global safety and security of shipping and ports;

e)	that operation of oceanographic radars provides benefits to society through environmental protection, disaster preparedness, public health protection, improved meteorological operations, increased coastal and maritime safety and enhancement of national economies;

f)	that oceanographic radars have been operated on an experimental basis around the world, providing an understanding of spectrum needs and spectrum sharing considerations, as well as an understanding of the benefits these systems provide;

g)	that performance and data requirements dictate the regions of spectrum that can be used by oceanographic radar systems for ocean observations,

recognizing

a)	that oceanographic radars have been operated  under provision No. 4.4 since the 1970s by several administrations;

b)	that developers of the experimental systems have implemented techniques to make the most efficient use of the spectrum and mitigate interference to other radio services;

c)	that for the purpose of protecting existing services from harmful interference, received interference by those services from oceanographic radars shall not exceed a power flux-density with an I/N ratio of −6 dB in an area where its radio noise is categorized to the rural and the quiet rural defined in Recommendation ITU-R P.372-10;

d)	that for the purpose of protecting existing services from harmful interference, oceanographic radars’ impact via ground-wave propagation can be checked by Report ITU-R M.[RLS 3-50 MHz SHARING] based on Recommendation ITU-R P.368-9,

resolves

1	that, when oceanographic radars are notified to the Bureau, the notification shall be in accordance with No. 11.2 of the RR and shall contain the station identification (call sign);

2	that the peak e.i.r.p. of an oceanographic radar shall not exceed 25 dBW;

3	that for transmission of the station identification machine readable code shall be used (No. 19.20);

4	that the separation distance1 between an oceanographic radar and the border of a neighbouring country should be higher than the distances specified in the following table unless prior coordination agreements between neighbouring countries are undertaken:

| Frequency (MHz) | Land path |  | Sea or mixed path |  |
| --- | --- | --- | --- | --- |
|  | Rural | Quiet rural | Rural | Quiet rural |
| 5 (± 1 MHz) | 120 | 170 | 790 | 920 |
| 9 (± 1 MHz) | 100 | 130 | 590 | 670 |
| 13 (± 1 MHz) | 100 | 110 | 480 | 520 |
| 16 (± 1 MHz) | 80 | 100 | 390 | 450 |
| 25 (± 3 MHz) | 80 | 100 | 280 | 320 |
| 42 (± 3 MHz) | 80 | 100 | 200 | 230 |

Reason: APT Members support new allocations to the RLS for oceanographic radar applications with appropriate regulatory provisions especially on station identification, emission power limitation, and separation distances.

____________

1	Given the difficulty in predicting the harmful interference from the sky-wave propagation the separation distance must be understood to be the minimum required separation distance beyond which an oceanographic radar will not cause harmful interference into co-primary service receivers via ground-wave propagation. The rural or quiet rural environment applies to the location of the fixed or mobile service receiver, not the radiolocation system location.