|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-21 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-03(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.21)

Agenda Item 1.21

1.21	to consider a primary allocation to the radiolocation service in the band 15.4–15.7 GHz, taking into account the results of ITU-R studies, in accordance with Resolution 614 (WRC-07).

Introduction

At the fifth meeting of the Conference Preparatory Group for WRC-12, APT Members, when considering Agenda Item 1.21, supported Method A of the CPM report.

The addition of a primary allocation for the RLS in the 15.4-15.7 GHz band and examples of new footnotes protecting ARNS and RAS systems are given below. In addition, it is possible that a WRC-12 Resolution (yet to be developed) may also be needed to provide further clarification.

The related proposals are given below.

Proposals

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD 		ASP/1.21/1

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511D |  |  |
| 15.43-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511C |  |  |
| 15.63-15.7 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511D |  |  |
| ... |  |  |

ADD	ASP/1.21/2

5.A121	In the band 15.4-15.7 GHz, stations operating in the radiolocation service shall not cause harmful interference to, nor claim protection from, stations operating in the aeronautical radionavigation service.

ADD	ASP/1.21/3

5.B121	In order to protect the radio astronomy service in the band 15.35-15.4 GHz, transmissions from radiolocation stations operating in the band 15.4-15.7 GHz shall not exceed the power flux-density level of −156 dB(W/m2) in a 50 MHz bandwidth into the band 15.35-15.4 GHz, at any radio astronomy observatory site for more than 2% of the time.

SUP	ASP/1.21/4

RESOLUTION 614 (WRC-07)

Use of the band 15.4-15.7 GHz by the radiolocation service

Reason: The addition of a primary allocation for the RLS in the 15.4-15.7 GHz band is no longer required. It is possible that a WRC-12 Resolution may also be needed to provide further clarification.

____________