|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-10 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-14(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.10)

Agenda Item 1.10

1.10	to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and associated regulatory provisions, in accordance with Resolution 357 (WRC-07)

Introduction

In order to satisfy this agenda item the APT Members consider that:

- With regards to regulatory status of AIS 1 and AIS 2 (Issue A), APT Members support Method A1 of CPM Report to WRC-12; which consists of the introduction of a primary allocation to the maritime mobile service and secondary allocations to the aeronautical mobile and mobile-satellite (Earth-to-space) services in the Table of Frequency Allocations in the bands 161.9625-161.9875 MHz and 162.0125-162.0375 MHz and suppressing RR No. 5.227A.

- Regarding satellite AIS (Issue B). APT Members support method B2 (Primary allocation for MSS (E-s) for channels 75 and 76 (via footnote) of the CPM Report for the use of the frequencies 156.775 MHz and 156.825 MHz (corresponding to channels 75 and 76 of RR Appendix 18) for improvement of the satellite detection of AIS (Automatic Identification System) emissions, broadcasting long range AIS messages.

- Regarding broadcasts of safety and security information for ships and ports (Issue C), APT Members support an exclusive primary allocation to the maritime mobile service in the band 495-505 kHz in all three regions and a co-primary allocation in the band 510-525 kHz in Region 2.  This corresponds to Method C of the draft CPM Report;

- Regarding the revision of Appendix 18 including VHF Data (Resolution 342 (Rev.WRC-2000)), port operations and ship movement, for ship/port security and maritime safety systems (Resolves 1 of Resolution 357 (WRC-07)) (Issue D), APT Members support a review of Appendix 18 for the identification of more single-frequency voice channels for port operation and ship movement. With regard to the VHF data channel(s), APT Members support Method D1 of CPM Report which consists of the introduction of a digital band(s) and identification of additional simplex channels in RR Appendix 18.

- The implementation date for the digital band(s) is proposed to be 1 January 2017 which gives 5 years for industry to react and users to migrate, if they wish.

Any modification under this agenda item should not affect the frequencies used by the GMDSS. The related proposals are given below.

Proposals

Introduction to Regulatory Status of AIS 1 and AIS 2 (Issue A)

APT Members fully support the Method A1 in the CPM Report to WRC-12. The following texts are exactly the same as the CPM Report Method A1.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD	ASP/1.10/1

148-223 MHz

| Editorial Note: In the Table above, if the proposed modifications for the bands 161.9625-161.9875 MHz and 162.0125-162.0375 MHz are accepted, then the frequency allocations in these bands become identical for all three Regions and the corresponding cells of the Table should be merged for Regions 1, 2 and 3. RR No. 5.229 will also be part of the merged cells for the frequency band 162.0125-162.0375 MHz. Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| … |  |  |  |  |
| - 156.8375-161.9625 - FIXED - MOBILE except aeronautical mobile |  | - 156.8375-161.9625 - FIXED - MOBILE |  |  |
| - 5.226 |  | - 5.226 |  |  |
| 161.9625-161.9875 MARITIME MOBILE Aeronautical mobile (OR) Mobile-satellite (Earth-to-space) |  | 161.9625-161.9875 MARITIME MOBILE Aeronautical mobile (OR) Mobile-satellite (Earth-to-space) |  |  |
| 5.226 ADD 5.A110 |  | 5.226 ADD 5.A110 |  |  |
| 161.9875-162.0125 - FIXED - MOBILE except aeronautical mobile |  | 161.9875-162.0125 FIXED MOBILE |  |  |
| 5.226 5.229 |  | 5.226 |  |  |
| 162.0125-162.0375 MARITIME MOBILE - Aeronautical mobile (OR) - Mobile-satellite (Earth-to-space) |  | - 162.0125-162.0375 MARITIME MOBILE Aeronautical mobile (OR) Mobile-satellite (Earth-to-space) |  |  |
| 5.226 5.229 ADD 5.A110 |  | 5.226 ADD 5.A110 |  |  |
| - 162.0375-174 FIXED MOBILE except aeronautical mobile |  | 162.0375-174 FIXED MOBILE |  |  |
| 5.226 5.229 |  | 5.226 5.230 5.231 5.232 |  |  |
| … |  | … |  | … |

Reason: Splits the bands for AIS 1 and AIS 2 for primary allocation to MMS.

ADD	ASP/1.10/2

5.A110	The use of the bands 161.9625-161.9875 MHz and 162.0125-162.0375 MHz by the mobile-satellite service (Earth-to-space) and the aeronautical mobile (OR) service is limited to automatic identification system (AIS) emissions operating in accordance with Appendix 18.   (WRC-12)

Reason: Limits the usage of the bands for AIS 1 and AIS 2.

SUP	ASP/1.10/3

5.227A

Reason: It is covered by 5.A110.

Introduction to Satellite AIS (Issue B)

APT Members fully support the Method B2 in the CPM Report to WRC-12. The following texts are exactly the same as the CPM Report Method B2.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	ASP/1.10/4

148-223 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |
| ... | ... |  |  |
| 156.7625-156.8375 MARITIME MOBILE (distress and calling) 5.111 5.226 ADD 5.D110 |  |  |  |
| ... |  | ... |  |

Reason: Allocates the bands for CH 75 and 76 to MSS by No. 5.D110.

ADD	ASP/1.10/5

5.D110	Additional allocation:  the bands 156.7625-156.7875 MHz and 156.8125-156.8375 MHz are also allocated to the mobile-satellite service (Earth-to-space) on a primary basis for the reception of automatic identification system (AIS) emissions, broadcasting long-range AIS message (Message 27, see the most recent version of Recommendation ITU-R M.1371), from stations operating in the maritime mobile service (see Appendix 18).     (WRC-12)

Reason: Allocates the bands for CH 75 and 76 to MSS on a primary basis.

MOD	ASP/1.10/6

APPENDIX 18 (Rev.WRC-12)

Table of transmitting frequencies in the
VHF maritime mobile band

(See Article 52)

NOTE A – For assistance in understanding the Table, see Notes a) to r) below.     (WRC-12)

NOTE B – The Table below defines the channel numbering for maritime VHF communications based on 25 kHz channel spacing and use of several duplex channels, but also allows the use of 12.5 kHz channel spacing. The channel numbering for 12.5 kHz channels and the conversion of two-frequency channels for single-frequency operation shall be in accordance with Recommendation ITU-R M.1084-4 Annex 4, Tables 1 and 3.     (WRC-07)

Reason: Adds Note r).

MOD	ASP/1.10/7

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n) r) | 156.775 | 156.775 |  | x |  |  |
| 16 | f ) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n) r) | 156.825 | 156.825 |  | x |  |  |

Reason: Allocates CHs 75 and 76 to MSS by adding Note r).

Notes referring to the Table

General notes

NOC

a) to e)

Specific notes

NOC

f) to q)

ADD	ASP/1.10/8

r)	Additionally, these channels (75 and 76) may be used by the mobile-satellite service (Earth-to-space) for the reception of long-range AIS broadcast messages from ships (Message 27, see the most recent version of Recommendation ITU-R M.1371).     (WRC-12)

Reason: Allocates CHs 75 and 76 to MSS on a primary basis.

Introduction to broadcasts of safety and security information for ships and ports (Issue C)

APT Members fully support the Method C in the CPM Report to WRC-12. The following texts are exactly the same as the CPM Report Method C.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	ASP/1.10/9

495-1 800 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 495-505 MARITIME MOBILE |  |  |
| 505-526.5 MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | 505-510 MARITIME MOBILE 5.79 | 505-526.5 MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile |
|  | 510-525 MARITIME MOBILE 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | Land mobile |
| 5.72 | ... |  |
| ... |  | ... |

Reason: Allocates 495-505 kHz to MMS for all regions on an exclusively primary basis and 510-525 kHz to MMS for Region 2 on co-primary basis.

SUP	ASP/1.10/10

5.82A

Reason: Removes the usage limitation.

SUP	ASP/1.10/11

5.82B

Reason: No longer required due to the allocation of 495-505 kHz to MMS on an exclusive primary basis.

Introduction to RR Appendix 18 (Issue D)

APT Members fully support the Method D1 in the CPM Report to WRC-12. The following texts are exactly the same as the CPM Report Method D1.

MOD	ASP/1.10/12

APPENDIX  18 (Rev.WRC-12)

Table of transmitting frequencies in the
VHF maritime mobile band

(See Article 52)

NOTE A – For assistance in understanding the Table 1, see Notes a) to FFF) below.     (WRC-12)

NOTE B – Table 1 below defines the channel numbering for maritime VHF communications based on 25 kHz channel spacing and use of several duplex channels. The channel numbering and the conversion of two-frequency channels for single-frequency operation shall be in accordance with Recommendation ITU-R M.1084-4 Annex 4, Tables 1 and 3. Table 1 below describes also the harmonized band where the digital technologies defined in Recommendation ITU-R M.1842 could be deployed.    (WRC-12)

NOTE C – Table 2 defines the channel numbering based on 25 kHz channel spacing for the digital band or for analogue usage, see Note CCC).

table 1

| Channel designator | Notes | Transmitting frequencies (MHz) |  |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations |  | From coast stations |  | Single frequency | Two frequency |  |
| 1060 | AAA) | 156.025 |  | 156.025 |  | x |  |  |
| 2060 | AAA) | 160.625 |  | 160.625 |  | x |  |  |
| 1001 | AAA) | 156.050 |  | 156.050 |  | x |  |  |
| 2001 | AAA) | 160.650 |  | 160.650 |  | x |  |  |
| 1061 | AAA) | 156.075 |  | 156.075 |  | x |  |  |
| 2061 | AAA) | 160.675 |  | 160.675 |  | x |  |  |
| 1002 | AAA) | 156.100 |  | 156.100 |  | x |  |  |
| 2002 | AAA) | 160.700 |  | 160.700 |  | x |  |  |
| 1062 | AAA) | 156.125 |  | 156.125 |  | x |  |  |
| 2062 | AAA) | 160.725 |  | 160.725 |  | x |  |  |
| 1003 | AAA) | 156.150 |  | 156.150 |  | x |  |  |
| 2003 | AAA) | 160.750 |  | 160.750 |  | x |  |  |
| 1063 | AAA) | 156.175 |  | 156.175 |  | x |  |  |
| 2063 | AAA) | 160.775 |  | 160.775 |  | x |  |  |
| 1004 | AAA) | 156.200 |  | 156.200 |  | x |  |  |
| 2004 | AAA) | 160.800 |  | 160.800 |  | x |  |  |
| 1064 | AAA) | 156.225 |  | 156.225 |  | x |  |  |
| 2064 | AAA) | 160.825 |  | 160.825 |  | x |  |  |
| 1005 | AAA) | 156.250 | 156.250 |  |  | x |  |  |
| 2005 | AAA) | 160.850 | 160.850 |  |  | x |  |  |
| 1065 | AAA) | 156.275 | 156.275 |  |  | x |  |  |
| 2065 | AAA) | 160.875 | 160.875 |  |  | x |  |  |
| 06 | f ) | 156.300 |  |  | x |  |  |  |
| 66 |  | 156.325 | 160.925 |  |  |  | x | x |
| 07 |  | 156.350 | 160.950 |  |  |  | x | x |
| 67 | h) | 156.375 | 156.375 |  | x | x |  |  |
| 08 |  | 156.400 |  |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  |  | x |  |  |
| 09 | i) | 156.450 | 156.450 |  | x | x |  |  |
| 69 |  | 156.475 | 156.475 |  | x | x |  |  |
| 10 | h), q ) | 156.500 | 156.500 |  | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 |  | Digital selective calling for distress, safety and calling |  |  |  |
| 11 | q ) | 156.550 | 156.550 |  |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  |  | x |  |  |
| 72 | i) | 156.625 |  |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 |  | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 |  | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  |  | x |  |  |
| 15 | g) | 156.750 | 156.750 |  | x | x |  |  |
| 75 | n) | 156.775 | 156.775 |  |  | x |  |  |
| 16 | f ) | 156.800 | 156.800 |  | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n) | 156.825 | 156.825 |  |  | x |  |  |
| 17 | g) | 156.850 | 156.850 |  | x | x |  |  |
| 77 |  | 156.875 |  |  | x |  |  |  |
| 18 |  | 156.900 | 161.500 |  |  |  | x | x |
|  |  |  |  |  |  | x | x | x |
| Digital band | BBB), CCC), DDD), FFF) | 156.925 to 157.325 | 161.525 to 161.925 |  |  |  | x |  |
| 27 |  | 157.350 | 161.950 |  |  |  | x | x |
| 87 |  | 157.375 | 157.375 |  |  | x |  |  |
| 28 |  | 157.400 | 162.000 |  |  |  | x | x |
| 88 |  | 157.425 | 157.425 |  |  | x |  |  |
| AIS 1 | f ), l), p) | 161.975 | 161.975 |  |  |  |  |  |
| AIS 2 | f ), l), p) | 162.025 | 162.025 |  |  |  |  |  |

Editorial note: The Rules of Procedure on RR Appendix 18 was included in the table above.

Notes referring to Tables 1 and 2

Reason: Assigns digital bands and additional simplex channels.

General notes

NOC

a) to e)

Specific notes

NOC

f) to l)

SUP	ASP/1.10/13

m)

Reason: No longer required.

NOC

n)

SUP	ASP/1.10/14

o)

Reason: No longer required by the digital bands assignments.

NOC

p) to q)

ADD	ASP/1.10/15

AAA)	Until 1 January 2017 the existing duplex channels can continue to be assigned. From that date no new coast station assignments for duplex mode are permitted. However existing duplex mode assignments may be preserved for coastal stations and retained for vessels, as shown in the Table below:

| Channel designator | Transmitting frequencies (MHz) |  |
| --- | --- | --- |
|  | From ship station | From coast station |
| 60 | 156.025 | 160.625 |
| 01 | 156.050 | 160.650 |
| 61 | 156.075 | 160.675 |
| 02 | 156.100 | 160.700 |
| 62 | 156.125 | 160.725 |
| 03 | 156.150 | 160.750 |
| 63 | 156.175 | 160.775 |
| 04 | 156.200 | 160.800 |
| 64 | 156.225 | 160.825 |
| 05 | 156.250 | 160.850 |
| 65 | 156.275 | 160.875 |

Reason: Allows channels 01-05 and 60-65 to use as duplex operation until 1 January 2017.

ADD	ASP/1.10/16

BBB)	Until 1 January 2017, this duplex band is used for analogue communications using the channelling arrangement described in Table 2.

Reason: Allows the bands for analogue communications until 1 January 2017.

ADD	ASP/1.10/17

CCC)	From 1 January 2017, the duplex band (156.925 to 157.325 MHz and 161.525 to 161.925 MHz) is identified for the utilization of the digital systems described in the most recent version of Recommendation ITU-R M.1842. This band could also be used for analogue modulation described in the most recent version of Recommendation ITU-R M.1084 for the administration that wishes to do so, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions. See Table 2 for the channel numbering based on 25 kHz channel spacing for the digital or for analogue usage.

Reason: Assigns the bands as the digital bands from 1 January 2017.

ADD	ASP/1.10/18

DDD)	In the United States and Canada, only the bands 157. 200-157.325 and 161.800-161.925 MHz (corresponding to the channels: 24, 84, 25, 85, 26 and 86; see Table 2) are authorized for digitally modulated emissions.

Reason: Limits the digital bands in United States and Canada.

ADD	ASP/1.10/19

EEE)	These channels may be operated as single-frequency channels, subject to coordination with affected administrations.

Reason: Allows the duplex channels to use as simplex operation subject to the coordination with affected administrations.

ADD	ASP/1.10/20

FFF)	In China, only the bands 157.150-157.325 and 161.750-161.925 MHz (corresponding to the channels: 23, 83, 24, 84, 25, 85, 26 and 86; see Table 2) are authorized for digitally modulated emissions.

Reason: Limits the digital bands in China.

ADD	ASP/1.10/21

Table 2

Channel numbering based on 25 kHz channel spacing for the digital band 
(narrow-band usage) or for analogue usage

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 78 | EEE) | 156.925 | 161.525 |  | x | x |  |
| 19 | EEE) | 156.950 | 161.550 |  | x | x |  |
| 79 | EEE) | 156.975 | 161.575 |  | x | x |  |
| 20 | EEE) | 157.000 | 161.600 |  | x | x |  |
| 80 | EEE) | 157.025 | 161.625 |  | x | x |  |
| 21 | EEE) | 157.050 | 161.650 |  | x | x |  |
| 81 | EEE) | 157.075 | 161.675 |  | x | x |  |
| 22 | EEE) | 157.100 | 161.700 |  | x | x |  |
| 82 | EEE) | 157.125 | 161.725 |  | x | x |  |
| 23 | EEE), FFF) | 157.150 | 161.750 |  | x | x |  |
| 83 | EEE), FFF) | 157.175 | 161.775 |  | x | x |  |
| 24 | EEE), FFF), DDD) | 157.200 | 161.800 |  | x | x |  |
| 84 | EEE), FFF), DDD) | 157.225 | 161.825 |  | x | x |  |
| 25 | EEE), FFF), DDD) | 157.250 | 161.850 |  | x | x |  |
| 85 | EEE), FFF), DDD) | 157.275 | 161.875 |  | x | x |  |
| 26 | EEE), FFF), DDD) | 157.300 | 161.900 |  | x | x |  |
| 86 | EEE), FFF), DDD) | 157.325 | 161.925 |  | x | x |  |

Reason: Specifies channel numbering for digital band or for analogue usage.

____________