|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-24 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-08(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.24)

Agenda Item 1.24

1.24	to consider the existing allocation to the meteorological-satellite service in the band 
7 750-7 850 MHz with a view to extending this allocation to the band 7 850–7 900 MHz, limited to non-geostationary meteorological satellites in the space-to-Earth direction, in accordance with Resolution 672 (WRC-07).

Introduction

The relevant ITU-R studies between non-geostationary MetSat in the band 7 850-7 900 MHz and the FS confirmed that sharing is feasible under the same regulatory conditions and using the same parameters as those currently applied in the band 7 750-7 850 MHz. Sharing with the mobile service (MS) systems would not be significantly different than that for FS systems and that similar pfd limits that would be applied to the FS in the band 7 850-7 900 MHz could also be applied to the MS.

APT Members support the single method outlined in the CPM Report to WRC-12, in accordance with the sections 4/1.24/5 and 4/1.24/6.

The related proposals are given below.

Proposals

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	ASP/1.24/1

7 250-8 500 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 7 750-7 900 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOD 5.461B MOBILE except aeronautical mobile |  |  |
| 7 |  |  |
| ... |  |  |

Reason: In the light of the ITU-R studies, the potential MetSat allocation in the band 7 850-7 900 MHz can be shared under the same conditions as the current allocation in the band 7 750-7 850 MHz.

MOD	ASP/1.24/2

5.461B	The use of the band 7 750-7 900 MHz by the meteorological-satellite service (space-to-Earth) is limited to non-geostationary satellite systems.     (WRC-12)

Reason: Consequential change due to the extension of the MetSat allocation by 50 MHz to 7 900 MHz.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

MOD	ASP/1.24/3

TABLE  21-4  (continued )

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0-5 | 5-25 | 25-90 |  |
| ... | ... | ... | ... | ... | ... |
| 4 500-4 800 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 7 250-7 900 MHz | Fixed-satellite (space-to-Earth) Meteorological-satellite (space-to-Earth) Mobile-satellite Space research | –152 | –152  0.5( – 5) | –142 | 4 kHz |
| ... | ... |  | ... |  | ... |

Reason:  In the light of the ITU-R studies, the limits that are currently applied to the existing MetSat allocation are also required to be applied to the extended band 7850-7900 MHz.

APPENDIX  7  (Rev.WRC-12)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	ASP/1.24/4

TABLE  8c

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteorological-satellite 7, 8 | Meteorological-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Fixed- satellite 9 | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 |  | 12.5-12.75 12 |  | 15.4-15.7 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Aeronau-tical radio-navigation | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  |  | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | – |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.003 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.0015 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 4 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 |  |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | –18 | –18 | 43 | 43 | 42 | 42 |  | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | –17 5 | –17 5 | –5 | –5 | 10 | 10 |  |  | –10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | –60 | –60 | –2 | –2 | –3 | –3 |  | –7 | –5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 |  | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27  106 | 27  106 |  |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | –151.2 |  |  | –125 | –125 | –154 11 | –142 | –220 | –216 |  |  | –131 | –131 |  |  |  |

Reason: In the light of the results of ITU-R studies, the list of parameters required for the determination of the coordination distance for a receiving earth station in the band 7750-7850 MHz are also required for the extended band 7850-7900 MHz.

SUP	ASP/1.24/5

RESOLUTION  672  (WRC-07)

Extension of the allocation to the meteorological-satellite
service in the band 7 750-7 850 MHz

Reason: Studies required by Resolution 672 (WRC-07) have been completed.

_______________