|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-23 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-09(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.23)

Agenda Item 1.23

1.23	to consider an allocation of about 15 kHz in parts of the band 415–526.5 kHz to the amateur service on a secondary basis, taking into account the need to protect existing services.

Introduction

At the 5th meeting of the APT preparatory group for WRC-12 APT Members, when considering agenda item 1.23, supported an allocation of about 15 kHz in parts of the band 415-526.5 kHz to the amateur service on a secondary basis, taking into account the need to protect existing services. In particular Method A (472–487 kHz) of the CPM report is supported provided that appropriate protection and regulatory provisions are in place.

The proposed changes to the Radio Regulations appear below.

Proposals

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD 	ASP/1.23/1

200-495 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 415-435 MARITIME MOBILE 5.79 AERONAUTICAL RADIONAVIGATION 5.72 | 415-472 MARITIME MOBILE 5.79 Aeronautical radionavigation 5.80 |  |
| 435-472 MARITIME MOBILE 5.79 Aeronautical radionavigation |  |  |
| 5.72 MOD 5.82 | 5.77 5.78 MOD 5.82 |  |
| 472-487 MARITIME MOBILE 5.79 Aeronautical radionavigation 5.80 Amateur 5.72 5.77 MOD 5.82 |  |  |
| 487-495 MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.72 5.77 MOD 5.82 |  |  |

MOD 	ASP/1.23/2

5.82	In the maritime mobile service, the frequency 490 kHz is to be used exclusively for the transmission by coast stations of navigational and meteorological warnings and urgent information to ships, by means of narrow-band direct-printing telegraphy. The conditions for use of the frequency 490 kHz are prescribed in Articles 31 and 52. In using the band 415-495 kHz for the aeronautical radionavigation service or the band 472-487 kHz for the amateur service, administrations shall ensure that no harmful interference is caused to the frequency 490 kHz.     (WRC-07).

____________