|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-18 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-34(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.18)

Agenda Item 1.18

1.18	to consider extending the existing primary and secondary radiodetermination-satellite service (space-to-Earth) allocations in the band 2 483.5-2500 MHz in order to make a global primary allocation, and to determine the necessary regulatory provisions based upon the results of ITU-R studies, in accordance with Resolution 613 (WRC-07).

Introduction

APT Members support extending the existing primary and secondary radiodetermination-satellite service (space-to-Earth) allocations in the band 2 483.5-2 500 MHz to make a global primary allocation.

The related proposals are given below.

Proposals

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD		ASP/1.18/1

2 170-2 520 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIODETERMINATION-SATELLITE (space-to-Earth) 5.398 Radiolocation | 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIOLOCATION RADIODETERMINATION- SATELLITE (space-to-Earth) 5.398 | 2 483.5-2 500 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A RADIOLOCATION RADIODETERMINATION-SATELLITE (space-to-Earth) 5.398 |
| 5.150 5.371 5.397 5.399 MOD 5.400 5.402 | 5.150 5.402 | 5.150 MOD 5.400 5.402 5.399 |
| ... |  |  |

Reason: In Region 1, RR No. 5.398 is moved from the bottom of the table to the associated new RDSS allocation, as it is the case in Region 2 and 3.

MOD		ASP/1.18/2

5.400	Different category of service:  iIn Angola, Australia, Bangladesh, Burundi, China, Eritrea, Ethiopia, India, Iran (Islamic Republic of), the Libyan Arab Jamahiriya, Lebanon, Liberia, Madagascar, Mali, Pakistan, Papua New Guinea, the Dem. Rep. of the Congo, the Syrian Arab Republic, Sudan, Swaziland, Togo and Zambia, the allocation of the band 2 483.5-2 500 MHz to the radiodetermination-satellite service (space-to-Earth) is on a primary basis (see No. 5.33), the use of RDSS systems for which their complete coordination information has been received by the Radiocommunication Bureau before [the end of WRC-12], while retaining the primary status that they had before WRC-12, shall continue to apply the procedure of No.9.21 with respect to the countries not listed in this footnote (See also the provisions of No. 7.4A).

Reason: These provisions will retain the regulatory status of the existing RDSS systems after the global upgrade allocation in this band.

NOC	ASP/1.18/3

APPENDIX 5 (Rev.WRC-07)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

# 1	Coordination thresholds for sharing between MSS (space-to-Earth) and terrestrial services in the same frequency bands, between non-GSO MSS feeder links (space-to-Earth) and terrestrial services in the same frequency bands

ANNEX 1

TABLE  5-2 (WRC-07)

| Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| ... |  |  |  |  |  |  |
| 2 483.5-2 500 | All cases | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 | −144 dB(W/m2) in 4 kHz and −1261 dB(W/m2) in 1 MHz | 0.65 |  |

Reason: The coordination threshold values for MSS should be kept unchanged to protect the terrestrial and satellite services. The coordination threshold for RDSS should be specified in a separate subsection.

APPENDIX 5 (Rev.WRC-07)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

ANNEX 1

MOD	ASP/1.18/4

TABLE 5--1 (continued)     (WRC-12)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … |  |
| No. 9.14 Non-GSO/ terrestrial, GSO/ terrestrial | A space station in a satellite network in the frequency bands for which a footnote refers to No. 9.11A or to No. 9.14, in respect of stations of terrestrial services where threshold(s) is (are) exceeded | 1) Frequency bands for which a footnote refers to No. 9.11A; or 2) 11.7-12.2 GHz (Region 2 GSO FSS) | i) See § 1 of Annex 1 to this Appendix; In the bands specified in No. 5.414A, the detailed conditions for the application of No. 9.14 are provided in No. 5.414A for MSS networks or ii) See § X of Annex 1 to this Appendix; for RDSS networks iii) In the band 11.7-12.2 GHz (Region 2 GSO FSS): –124 dB(W/(m2 · MHz)) for 0°    5 –124 + 0.5 ( – 5) dB(W/(m2 · MHz)) for 5° <   25 –114 dB(W/(m2 · MHz)) for  > 25 where  is the angle of arrival of the incident wave above the horizontal plane (degrees) | i) See § 1 of Annex 1 to this Appendix |  |

APPENDIX 5 (Rev.WRC-07)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

ANNEX 1

ADD	ASP/1.18/5

- X	Coordination thresholds for sharing between RDSS (space-to-Earth) and terrestrial services in the same frequency bands

- X.1	Band 2 483.5-2 500 MHz

- X.1.1	Objectives

Generally, pfd thresholds were used to determine the need for coordination between space stations of the RDSS (space-to-Earth) and terrestrial services.

- X.1.2	In the band 2 483.5-2 500 MHz, coordination of a space station of the RDSS (space-to-Earth) systems with respect to terrestrial services is not required if the pfd produced at the Earth’s surface does not exceed the threshold values shown in the following table.

TABLE 5-3 (WRC-12)

| Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| 2 483.5-2 500 [NOTE A118] | All cases, except radiolocation services | −128 dB(W/m2) in 1 MHz |  | −129 dB(W/m2) in 1 MHz |  |  |

NOTE A118 – These pfd values do not apply to systems for which their complete coordination information has been received before [the end of WRC-12]. (See No. 5.400)

NOTE 1 – Refer to NOTE 1 in Table 5-2.

NOTE 2 – Refer to NOTE 2 in Table 5-2.

SUP	ASP/1.18/6

RESOLUTION  613  (WRC-07)

Global primary allocation to the radiodetermination-satellite service
in the frequency band 2 483.5-2 500 MHz (space-to-Earth)

Reason: Action under this Resolution has been completed.

__________________