|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-09 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-13(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.9)

Agenda Item 1.9

1.9	to revise frequencies and channelling arrangements of Appendix 17 to the Radio Regulations, in accordance with Resolution 351 (Rev.WRC-07), in order to implement new digital technologies for the maritime mobile service;

Introduction

APT Members support Method A2 in the CPM Report to WRC-12.

The texts proposed are almost the same as the text in Method A2 (1/1.9/6.2), with the only changes being the addition of Note 4 in Section VI, associated annotations for affected channels in the Table in Section VI, editorial correction of Note 3 on Section VI Table, and modification of Notes p) and cc).

The related proposals are given below.

Proposals

MOD 	ASP/1.9/1

APPENDIX  17  (Rev.WRC-12)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

PART  A  –  Table of subdivided bands     (WRC-12)

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies ( f.) and the spacing in kHz being indicated in italics.

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz allocated exclusively to the maritime mobile service (continued) Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) gg) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation a) gg) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| -Frequencies assignable to ship stations for data transmissions p) ee) | 4 154.5 to 4 169.5 6 f. 3 kHz | 6 235 to 6 259 9 f. 3 kHz | 8 302 to 8 338 13 f. 3 kHz | 12 370 to 12 418 17 f. 3 kHz | 16 551.5 to 16 614.5 22 f. 3 kHz | 18 847.5 to 18 868.5 8 f. 3 kHz | 22 181.5 to 22 238.5 20 f. 3 kHz | 25 123 to 25 138 6 f. 3 kHz |
| Limits (kHz) | 4 172 | 6 261 | 8 340 | 12 420 | 16 617 | 18 870 | 22 240 | 25 139.5 |
| Frequencies assignable to ship stations for oceanographic data transmission c) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 139.5 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  | 18 871.5 1 f. 3 kHz |  |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 873.25 | 22 241.75 | 25 139.5 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) | 4 172.5 to 4 178 12 f. 0.5 kHz | 6 263 to 6 268.5 12 f. 0.5 kHz |  |  |  | 18 873.5 to 18 880 14 f. 05 kHz |  |  |
| Limits (kHz) | 4 178.25 | 6 268.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 880.75 | 22 241.75 | 25 139.5 |
| Limits (kHz) | 4 178.25 | 6 268.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 880.75 | 22 241.75 | 25 139.5 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) | 4 180 1 f. 3 kHz | 6 270.5 to 6 273.5 2 f. 3 kHz |  |  |  | 18 883.5 to 18 886.5 2 f. 3 kHz |  |  |
| Limits (kHz) | 4 181.75 | 6 275 | 8 341.75 | 12 421.75 | 16 618.75 | 18 889 | 22 241.75 | 25 139.5 |
| Frequencies assignable to ship stations for data transmission m) p) | 4 183.25 1 f. 3 kHz | 6 276.5 to 6 279.5 2 f. 3 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 184.75 | 6 281 | 8 341.75 | 12 421.75 | 16 618.75 | 18 889 | 22 241.75 | 25 139.5 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  | 6 282.5 1 f. 3 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 184.75 | 6 284 | 8 341.75 | 12 421.75 | 16 618.75 | 18 889 | 22 241.75 | 25 139.5 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 184.75 | 6 284 | 8 341.75 | 12 421.75 | 16 618.75 | 18 889 | 22 241.75 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission m) p) | 4 186.25 to 4 201.25 6 f. 3 kHz | 6 285.5 to 6 297.5 5 f. 3 kHz | 8 343.25 to 8 364.25 8 f. 3 kHz | 12 423.75 to 12 474.75 18 f. 3 kHz | 16 621 to 16 681 21 f. 3 kHz |  | 22 243.5 to 22 276.5 12 f. 3 kHz |  | (WRC-07) |
| Limits (kHz) | 4 202.75 | 6 299 | 8 365.75 | 12 476.75 | 16 683.25 | 18 889 | 22 278 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission m) p) |  |  | 8 368 to 8 374 3 f. 3 kHz |  |  |  | 22 279.5 to 22 282.5 2 f. 3 kHz |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 376.25 | 12 476.75 | 16 683.25 | 18 889 | 22 284.25 | 25 139.5 |  |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) |  |  | 8 376.5 to 8 383.5 15 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 383.75 | 12 476.75 | 16 683.25 | 18 889 | 22 284.25 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  | 8 385.5 to 8 394.5 4 f. 3 kHz |  |  |  | 22 286 to 22 289 2 f. 3 kHz |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 476.75 | 16 683.25 | 18 889 | 22 290.75 | 25 139.5 |  |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) |  |  |  | 12 477 to 12 522.5 92 f. 0.5 kHz | 16 683.5 to 16 698.5 31 f. 0.5 kHz |  | 22 291 to 22 297 13 f. 0.5 kHz |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 522.75 | 16 698.75 | 18 889 | 22 297.25 | 25 139.5 |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 522.75 | 16 698.75 | 18 889 | 22 297.25 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  | 12 524.25 to 12 548.25 9 f. 3 kHz | 16 700.5 to 16 733.5 12 f. 3 kHz |  | 22 299.5 to 22 350.5 18 f. 3 kHz |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 549.75 | 16 735 | 18 889 | 22 352.5 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  | 12 551.25 to 12 554.25 2 f. 3 kHz | 16 736.5 to 16 739.5 2 f. 3 kHz |  |  |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 555.75 | 16 741 | 18 889 | 22 352.5 | 25 139.5 |  |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  | 12 557.25 to 12 560.25 2 f. 3 kHz | 16 742.5 to 16 784.5 15 f. 3 kHz |  |  |  |  |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 561.75 | 16 786 | 18 889 | 22 352.5 | 25 139.5 |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.75 | 6 299 | 8 396.25 | 12 561.75 | 16 786 | 18 889 | 22 352.5 | 25 139.5 |
| Frequencies (non paired) assignable to ship stations for data transmission systems b) m) p) dd) | 4 205 1 f. 3 kHz | 6 300.5 to 6 309.5 4 f. 3 kHz | 8 397.75 to 8 412.75 6 f. 3 kHz | 12 563.25 to 12 575.25 5 f. 3 kHz | 16 787.5 to 16 802.5 6 f. 3 kHz | 18 890.5 to 18 896.5 3 f. 3 kHz | 22 354 to 22 372 7 f. 3 kHz | 25 142 to 25 205 22 f. 3 kHz |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414.25 | 12 576.75 | 16 804.25 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) |  | 6 314 1 f. 0.5 kHz |  |  |  | 19 680.5 1 f. 0.5 kHz | 22 376 1 f. 0.5 kHz | 26 100.5 1 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 314.25 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.75 | 22 376.25 | 26 100.75 |
| Frequencies assignable to coast stations for data transmission n) o) p) aa) bb) cc) |  |  |  |  |  | 19 682.25 1 f. 3 kHz | 22 378 to 22 381 2 f. 3 kHz | 26 103.5 to 26 118.5 6 f. 3 kHz |
| Limits (kHz) | 4 209.25 | 6 314.25 | 8 416.25 | 12 578.75 | 16 806.25 | 19 683.75 | 22 382.75 | 26 120.75 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) n) o) | 4 209.5 to 4 215.5 13 f. 0.5 kHz | 6 314.5 to 6 319.5 12 f. 0.5 kHz | 8 416.5 to 8 423.5 15 f. 0.5 kHz | 12 579 to 12 624.5 93 f. 0.5 kHz | 16 806.5 to 16 821.5 32 f. 0.5 kHz | 19 684 to 19 690.5 14 f. 0.5 kHz | 22 383 to 22 389 13 f. 0.5 kHz |  |
| Limits (kHz) | 4 215.75 | 6 319.75 | 8 423.75 | 12 624.75 | 16 821.75 | 19 690.75 | 22 389.25 | 26 120.75 |
| Limits (kHz) | 4 215.75 | 6 319.75 | 8 423.75 | 12 624.75 | 16 821.75 | 19 690.75 | 22 389.25 | 26 120.75 |
| Frequencies assignable to coast stations for data transmission d) p) aa) bb) cc) | 4 217.5 1 f. 3 kHz | 6 322 to 6 328 3 f. 3 kHz | 8 425.5 to 8 434.5 4 f. 3 kHz | 12 627 to 12 654 10 f. 3 kHz | 16 823.25 to 16 901.25 27 f. 3 kHz |  |  |  |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 690.75 | 22 389.25 | 26 120.75 |
| Frequencies assignable to coast stations for data transmission n) o) p) aa) bb) cc) |  |  |  |  |  | 19 692.5 to 19 701.5 4 f. 3 kHz | 22 391 to 22 442 18 f. 3 kHz |  |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz | 22 444 to 22 445 3 f. 0.5 kHz | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for data transmission d) p) aa) bb) cc) | 4 222.5 to 4 258.5 13 f. 3 kHz | 6 334 to 6 379 16 f. 3 kHz | 8 439.5 to 8 508.5 24 f. 3 kHz | 12 660 to 12 771 38 f. 3 kHz | 16 906 to 17 038 45 f. 3 kHz |  |  |  |
| Limits (kHz) | 4 260 | 6 380.5 | 8 510 | 12 772.5 | 17 039.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for data transmission n) o) p) aa) bb) cc) |  |  |  |  |  | 19 706.5 to 19 721.5 6 f. 3 kHz | 22 447 to 22 546 34 f. 3 kHz |  |
| Limits (kHz) | 4 260 | 6 380.5 | 8 510 | 12 772.5 | 17 039.5 | 19 723 | 22 447.5 | 26 122.5 |
| Frequencies assignable to coast stations for wide-band, facsimile, special and data transmission systems and direct-printing telegraphy systems m) p) ee) ff) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) gg) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

Reasons:  Enables to assign spectrums to the new digital technologies based on 3 kHz bandwidth under some conditions. The frequency boundaries are adjusted for adequate alignment of 3 kHz bandwidth channels, taking also into account the paired channel assignments to ship and coast stations.

NOC

a)

MOD	ASP/1.9/2

b)	Until 1 January 2017, see Part B, Section III. After this date, Section III will no longer apply and has to be deleted by a future competent WRC.

Reason:	Deletes NBDP simplex channels (Part B, Section III) after transition period.

NOC

c) to d)

SUP 	ASP/1.9/3

e)

Reason:	A1A Morse channel assignments are no longer required.

SUP	ASP/1.9/4

f)

Reason:	A1A Morse channel assignments are no longer required.

SUP	ASP/1.9/5

g)

Reason:	A1A Morse channel assignments are no longer required.

NOC

h) to l)

MOD	ASP/1.9/6

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

Reason: Allows A1A or A1B Morse telegraphy to continuously use the bands subject to not claiming protection.

NOC

n) and o)

MOD	ASP/1.9/7

p)	These sub-bands, except the frequencies referred to in Notes i), j), n) and o), may be used for digitally modulated emissions for maritime mobile service (e.g. as described in Recommendation ITU-R M.1798). The provisions of No. 15.8 apply.

Reason:	Allows the new digital technologies to use the bands except for the MSI frequencies.

ADD	ASP/1.9/8

aa)	Until 1 January 2017, these bands may be used by narrow-band direct-printing applications. Before 1 January 2017, administrations who introduce digitally modulated emissions for radiocommunications are urged to take all practicable steps to prevent interference to the narrow-band direct-printing applications in the band.

Reason:	Limits the use of the bands by NBDP until 1 January 2017.

ADD	ASP/1.9/9

bb)	From 1 January 2017 these bands, except the frequencies referred to in Notes n) and o), may be used by narrow-band direct-printing applications by administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

Reason: Allows NBDP to continuously use the bands even after 1 January 2017 subject to not claiming protection.

ADD	ASP/1.9/10

cc)	From 1 January 2017, administrations that make assignments to stations using digitally modulated emissions are encouraged to effect coordination with potentially affected administrations.

Reason: Requires coordination prior to assign the new digital technologies after 1 January 2017.

ADD	ASP/1.9/11

dd)	These bands may be used by narrow-band direct-printing applications by the administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

Reason: Allows NBDP to continuously use the bands subject to not claiming protection.

ADD	ASP/1.9/12

ee)	Frequencies from these bands may be used for wideband telegraphy, facsimile, A1A/A1B Morse telegraphy and special data transmission on the condition that interference is not caused to and protection is not claimed from stations in the maritime mobile service using digitally modulated emissions.

Reason: Allows wideband telegraphy, facsimile, A1A/A1B Morse telegraphy and special data transmission to continuously use the bands subject to not claiming protection.

ADD	ASP/1.9/13

ff)	The bands 4 345-4 351 kHz, 6 495-6 501 kHz, 8 701-8 707 kHz may be used for simplex (single-sideband) telephone operation (regularly spaced by 3 kHz), in accordance with provision No. 52.177, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

Reason: Allows single sideband simplex operation to use the bands subjects to not claiming protection.

ADD	ASP/1.9/14

gg) 	The bands 4 066.4-4 150.4 kHz, 4 352.4-4 436.4 kHz, 6 201.4-6 231.4 kHz, 6 502.4-6 523.4 kHz, 8 196.4-8 298.4 kHz, 8 708.4-8 813.4 kHz, 12 231.4-12 366.4 kHz, 13 078.4-13 198.4 kHz, 16 361.4-16  574.4 kHz, 17 243.4-17 408.4 kHz, 18 781.4-18 844.4 kHz, 19 756.4-19 798.4 kHz, 22 001.4-22 178.4 kHz, 22 697.4-22 853.4 kHz, 25 071.4-25 119.4 kHz, 26 146.4-26 173.4 kHz may be used, in accordance with the Appendix 25 allotment Plan, for digitally modulated emissions as described in Recommendation ITU-R M.1798 on condition that it shall not cause harmful interference to, or claim protection from other stations in the maritime mobile service using radiotelephony operations. The digitally modulated emissions may be used provided that their occupied bandwidth does not exceed 2 800 Hz, it is situated wholly within one frequency channel and the peak envelope power of coast stations does not exceed 10 kW and the peak envelope power of ship stations does not exceed 1.5 kW per channel.

Reason: Allows the new digital technologies to use the Appendix 25 bands subject to not claiming protection.

MOD	ASP/1.9/15

PART  B  –  Channelling arrangements     (WRC-12)

NOC

Section I  –  Radiotelephony

MOD	ASP/1.9/16

Section II  –  Narrow-band direct-printing telegraphy (paired frequencies)

1	Each coast station which uses paired frequencies is assigned one or more frequency pairs from the following series; each pair consists of a transmitting and a receiving frequency.

2	The speed of the narrow-band direct-printing telegraphy and data systems shall not exceed 100 Bd for FSK and 200 Bd for PSK.

Table of frequencies for two-frequency operation by coast stations (kHz)
to be used until 1 January 2017

Reason:	Splits the NBDP paired frequencies for before and after transition date.

NOC

The table itself is unchanged.

MOD	ASP/1.9/17

Table of frequencies for two-frequency operation by coast stations (kHz)
to be used after 1 January 2017

| Channel |  | 4 MHz band 1 |  | 6 MHz band |  | 8 MHz band |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  | Transmit | Receive | Transmit | Receive | Transmit | Receive |  |
| 1 2 3 4 5 |  | 4 210.5 4 211 4 211.5 4 212 4 212.5 | 4 172.5 4 173 4 173.5 4 174 4 174.5 | 6 314.5 6 315 6 315.5 6 316 6 316.5 | 6 263 6 263.5 6 264 6 264.5 6 265 | 8 376.5 2 8 417 8 417.5 8 418 8 418.5 | 8 376.5 2 8 377 8 377.5 8 378 8 378.5 |  |
| 6 7 8 9 10 |  | 4 213 4 213.5 4 214 4 214.5 4 215 | 4 175 4 175.5 4 176 4 176.5 4 177 | 6 317 6 317.5 6 318 6 318.5 6 319 | 6 265.5 6 266 6 266.5 6 267 6 267.5 | 8 419 8 419.5 8 420 8 420.5 8 421 | 8 379 8 379.5 8 380 8 380.5 8 381 |  |
| 11 12 13 14 15 |  | 4 177.5 2 4 215.5 | 4 177.5 2 4 178 | 6 268 2 6 319.5 | 6 268 2 6 268.5 | 8 421.5 8 422 8 422.5 8 423 8 423.5 | 8 381.5 8 382 8 382.5 8 383 8 383.5 |  |
|  | 1 Ship stations may use the coast station receiving frequencies for transmitting A1A or A1B Morse telegraphy (working), with the exception of channel No. 11 (see Appendix 15). 2 For the conditions of use of this frequency, see Article 31. |  |  |  |  |  |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel |  | 12 MHz band |  | 16 MHz band |  | 18/19 MHz band |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  | Transmit | Receive | Transmit | Receive | Transmit | Receive |  |
| 1 2 3 4 5 |  | 12 579.5 12 580 12 580.5 12 581 12 581.5 | 12 477 12 477.5 12 478 12 478.5 12 479 | 16 807 16 807.5 16 808 16 808.5 16 809 | 16 683.5 16 684 16 684.5 16 685 16 685.5 |  |  |  |
| 6 7 8 9 10 |  | 12 582 12 582.5 12 583 12 583.5 12 584 | 12 479.5 12 480 12 480.5 12 481 12 481.5 | 16 809.5 16 810 16 810.5 16 811 16 811.5 | 16 686 16 686.5 16 687 16 687.5 16 688 | 19 684 19 684.5 19 685 19 685.5 | 18 873.5 18 874 18 874.5 18 875 |  |
| 11 12 13 14 15 |  | 12 584.5 12 585 12 585.5 12 586 12 586.5 | 12 482 12 482.5 12 483 12 483.5 12 484 | 16 812 16 812.5 16 813 16 813.5 16 814 | 16 688.5 16 689 16 689.5 16 690 16 690.5 | 19 686 19 686.5 19 687 19 687.5 19 688 | 18 875.5 18 876 18 876.5 18 877 18 877.5 |  |
| 16 17 18 19 20 |  | 12 587 12 587.5 12 588 12 588.5 12 589 | 12 484.5 12 485 12 485.5 12 486 12 486.5 | 16 814.5 16 815 16 815.5 16 816 16 816.5 | 16 691 16 691.5 16 692 16 692.5 16 693 | 19 688.5 19 689 19 689.5 19 690 19 690.5 | 18 878 18 878.5 18 879 18 879.5 18 880 |  |
| 21 22 23 24 25 |  | 12 589.5 12 590 12 590.5 12 591 12 591.5 | 12 487 12 487.5 12 488 12 488.5 12 489 | 16 817 16 817.5 16 818 16 695 2 16 818.5 | 16 693.5 16 694 16 694.5 16 695 2 16 695.5 |  |  |  |
| 26 27 28 29 30 |  | 12 592 12 592.5 12 593 12 593.5 12 594 | 12 489.5 12 490 12 490.5 12 491 12 491.5 | 16 819 16 819.5 16 820 16 820.5 16 821 | 16 696 16 696.5 16 697 16 697.5 16 698 |  |  |  |
| 31 32 33 34 35 |  | 12 594.5 12 595 12 595.5 12 596 12 596.5 | 12 492 12 492.5 12 493 12 493.5 12 494 | 16 821.5 | 16 698.5 |  |  |  |
| 36 37 38 39 40 |  | 12 597 12 597.5 12 598 12 598.5 12 599 | 12 494.5 12 495 12 495.5 12 496 12 496.5 |  |  |  |  |  |
| 41 42 43 44 45 |  | 12 599.5 12 600 12 600.5 12 601 12 601.5 | 12 497 12 497.5 12 498 12 498.5 12 499 |  |  |  |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 12 MHz band (cont.) |  | 16 MHz band (cont.) |  |
| --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |
| 46 47 48 49 50 | 12 602 12 602.5 12 603 12 603.5 12 604 | 12 499.5 12 500 12 500.5 12 501 12 501.5 |  |  |
| 51 52 53 54 55 | 12 604.5 12 605 12 605.5 12 606 12 606.5 | 12 502 12 502.5 12 503 12 503.5 12 504 |  |  |
| 56 57 58 59 60 | 12 607 12 607.5 12 608 12 608.5 12 609 | 12 504.5 12 505 12 505.5 12 506 12 506.5 |  |  |
| 61 62 63 64 65 | 12 609.5 12 610 12 610.5 12 611 12 611.5 | 12 507 12 507.5 12 508 12 508.5 12 509 |  |  |
| 66 67 68 69 70 | 12 612 12 612.5 12 613 12 613.5 12 614 | 12 509.5 12 510 12 510.5 12 511 12 511.5 |  |  |
| 71 72 73 74 75 | 12 614.5 12 615 12 615.5 12 616 12 616.5 | 12 512 12 512.5 12 513 12 513.5 12 514 |  |  |
| 76 77 78 79 80 | 12 617 12 617.5 12 618 12 618.5 12 619 | 12 514.5 12 515 12 515.5 12 516 12 516.5 |  |  |
| 81 82 83 84 85 | 12 619.5 12 620 12 620.5 12 621 12 621.5 | 12 517 12 517.5 12 518 12 518.5 12 519 |  |  |
| 86 87 88 89 90 | 12 622 12 520 2 12 622.5 12 623 12 623.5 | 12 519.5 12 520 2 12 520.5 12 521 12 521.5 |  |  |
| 91 92 | 12 624 12 624.5 | 12 522 12 522.5 |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel |  | 22 MHz band |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. |  | Transmit | Receive | Transmit |  |  |
| 14 15 |  | 22 383 22 383.5 | 22 291 22 291.5 |  |  |  |
| 16 17 18 19 20 |  | 22 384 22 384.5 22 385 22 385.5 22 386 | 22 292 22 292.5 22 293 22 293.5 22 294 |  |  |  |
| 21 22 23 24 25 |  | 22 386.5 22 387 22 387.5 22 388 22 388.5 | 22 294.5 22 295 22 295.5 22 296 22 296.5 |  |  |  |
| 26 |  | 22 389 | 22 297 |  |  |  |

Reason: Splits the NBDP paired frequencies for before and after transition date.

MOD	ASP/1.9/18

Section III  –  Narrow-band direct-printing telegraphy 
(non-paired frequencies)
until 1 January 2017 (after this date the entire section will no longer 
apply and has to be deleted by a future competent WRC)

Reason: Keeps the existing NBDP non-paired frequencies until transition date.

SUP	ASP/1.9/19

Section IV  –  Morse telegraphy (calling)

Reason: Morse frequencies list is no longer required.

SUP	ASP/1.9/20

Section V  –  Morse telegraphy (working)

Reason: Morse frequencies list is no longer required.

ADD	ASP/1.9/21

Section VI  –  Data transmission

Table of frequencies (kHz) assignable to ship and coast stations
for data transmission (kHz)1 to be used from 1 January 2017

| Channel No. | 4 MHz band |  | 6 MHz band |  | 8 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coast TX (Ship RX) | Coast RX (Ship TX) | Coast TX (Ship RX) | Coast RX (Ship TX) | Coast TX (Ship RX) | Coast RX (Ship TX) |
| 1 2 3 4 5 | 4 205 2 4 217.5 4 4 222.5 3 4 225.5 3 4 228.5 3 | 4 205 2 4 154.5 4 4 157.5 3 4 160.5 3 4 163.5 3 | 6 300.5 2 6 303.5 2 6 306.5 2 6 309.5 2 6 322 3 | 6 300.5 2 6 303.5 2 6 306.5 2 6 309.5 2 6 235 3 | 8 397.75 2 8 400.75 2 8 403.75 2 8 406.75 2 8 409.75 2 | 8 397.75 2 8 400.75 2 8 403.75 2 8 406.75 2 8 409.75 2 |
| 6 7 8 9 10 | 4 231.5 3 4 234.5 3 4 237.5 4 4 240.5 4 4 243.5 3 | 4 166.5 3 4 169.5 3 4 180 4 4 183.25 4 4 186.25 3 | 6 325 3 6 328 3 6 334 4 6 337 4 6 340 4 | 6 238 3 6 241 3 6 244 4 6 247 4 6 250 4 | 8 412.75 2 8 425.5 3 8 428.5 3 8 431.5 3 8 434.5 3 | 8 412.75 2 8 302 3 8 305 3 8 308 3 8 311 3 |
| 11 12 13 14 15 | 4 246.5 3 4 249.5 3 4 252.5 3 4 255.5 3 4 258.5 3 | 4 189.25 3 4 192.25 3 4 195.25 3 4 198.25 3 4 201.25 3 | 6 343 4 6 346 4 6 349 4 6 352 4 6 355 4 | 6 253 4 6 256 4 6 259 4 6 270.5 4 6 273.5 4 | 8 439.5 4 8 442.5 4 8 445.5 4 8 448.5 4 8 451.5 4 | 8 314 4 8 317 4 8 320 4 8 323 4 8 326 4 |
| 16 17 18 19 20 |  |  | 6 358 4 6 361 4 6 364 4 6 367 3 6 370 3 | 6 276.5 4 6 279.5 4 6 282.5 4 6 285.5 3 6 288.5 3 | 8 454.5 4 8 457.5 4 8 460.5 4 8 463.5 4 8 466.5 3 | 8 329 4 8 332 4 8 335 4 8 338 4 8 343.25 3 |
| 21 22 23 24 25 |  |  | 6 373 3 6 376 3 6 379 3 | 6 291.5 3 6 294.5 3 6 297.5 3 | 8 469.5 3 8 472.5 3 8 475.5 3 8 478.5 3 8 481.5 3 | 8 346.25 3 8 349.25 3 8 352.25 3 8 355.25 3 8 358.25 3 |
| 26 27 28 29 30 |  |  |  |  | 8 484.5 3 8 487.5 3 8 490.5 4 8 493.5 4 8 496.5 4 | 8 361.25 3 8 364.25 3 8 368 4 8 371 4 8 374 4 |
| 31 32 33 34 |  |  |  |  | 8 499.5 3 8 502.5 3 8 505.5 3 8 508.5 3 | 8 385.5 3 8 388.5 3 8 391.5 3 8 394.5 3 |
| 1 The data transmission shall be in accordance with the latest version of Recommendation ITU-R M.1798. 2 Non-paired (simplex) operations only. 3 Assignable for wideband operation using multiples of 3 kHz bandwidth. 4 Assignable for duplex data transmission on a single channel 3 kHz bandwidth basis. |  |  |  |  |  |  |

Table of frequencies (kHz) assignable to ship and coast stations
for data transmission (kHz)1 to be used from 1 January 2017

| Channel No. | 12 MHz |  | 16 MHz |  | 18/19 MHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coast TX (Ship RX) | Coast RX (Ship TX) | Coast TX (Ship RX) | Coast RX (Ship TX) | Coast TX (Ship RX) | Coast RX (Ship TX) |
| 1 2 3 4 5 | 12 563.25 2 12 566.25 2 12 569.25 2 12 572.25 2 12 575.25 2 | 12 563.25 2 12 566.25 2 12 569.25 2 12 572.25 2 12 575.25 2 | 16 787.5 2 16 790.5 2 16 793.5 2 16 796.5 2 16 799.5 2 | 16 787.5 2 16 790.5 2 16 793.5 2 16 796.5 2 16 799.5 2 | 18 890.5 2 18 893.5 2 18 896.5 2 19 682.25 4 19 692.5 3 | 18 890.5 2 18 893.5 2 18 896.5 2 18 847.5 4 18 850.5 3 |
| 6 7 8 9 10 | 12 ,627 3 12 630 3 12 633 3 12 636 3 12 639 3 | 12 370 3 12 373 3 12 376 3 12 379 3 12 382 3 | 16 802.5 2 16 823.25 3 16 826.25 3 16 829.25 3 16 832.25 3 | 16 802.5 2 16 551.5 3 16 554.5 3 16 557.5 3 16 560.5 3 | 19 695.5 3 19 698.5 3 19 701.5 3 19 706.5 4 19 709.5 4 | 18 853.5 3 18 856.5 3 18 859.5 3 18 862.5 4 18 865.5 4 |
| 11 12 13 14 15 | 12 642 3 12 645 3 12 648 3 12 651 3 12 654 3 | 12 385 3 12 388 3 12 391 3 12 394 3 12 397 3 | 16 835.25 3 16 838.25 3 16 841.25 3 16 844.25 3 16 847.25 3 | 16 563.5 3 16 566.5 3 16 569.5 3 16 572.5 3 16 575.5 3 | 19 712.5 4 19 715.5 4 19 718.5 4 19 721.5 4 | 18 868.5 4 18 871.5 4 18 883.5 4 18 886.5 4 |
| 16 17 18 19 20 | 12 660 4 12 663 4 12 666 4 12 669 4 12 672 4 | 12 400 4 12 403 4 12 406 4 12 409 4 12 412 4 | 16 850.25 3 16 853.25 3 16 856.25 3 16 859.25 3 16 862.25 3 | 16 578.5 3 16 581.5 3 16 584.5 3 16 587.5 3 16 590.5 3 |  |  |
| 21 22 23 24 25 | 12 675 4 12 678 4 12 681 3 12 684 3 12 687 3 | 12 415 4 12 418 4 12 423.75 3 12 426.75 3 12 429.75 3 | 16 865.25 3 16 868.25 3 16 871.25 3 16 874.25 3 16 877.25 3 | 16 593.5 3 16 596.5 3 16 599.5 3 16 602.5 3 16 605.5 3 |  |  |
| 26 27 28 29 30 | 12 690 3 12 693 3 12 696 3 12 699 4 12 702 4 | 12 432.75 3 12 435.75 3 12 438.75 3 12 441.75 4 12 444.75 4 | 16 880.25 3 16 883.25 3 16 886.25 3 16 889.25 4 16 892.25 3 | 16 608.5 3 16 611.5 3 16 614.5 3 16 621 4 16 624 3 |  |  |
| 31 32 33 34 35 | 12 705 4 12 708 4 12 711 3 12 714 3 12 717 3 | 12 447.75 4 12 450.75 4 12 453.75 3 12 456.75 3 12 459.75 3 | 16 895.25 3 16 898.25 3 16 901.25 3 16 906 4 16 909 4 | 16 627 3 16 630 3 16 633 3 16 636 4 16 639 4 |  |  |
| 36 37 38 39 40 | 12 720 3 12 723 3 12 726 3 12 729 3 12 732 3 | 12 462.75 3 12 465.75 3 12 468.75 3 12 471.75 3 12 474.75 3 | 16 912 4 16 915 4 16 918 4 16 921 4 16 924 4 | 16 642 4 16 645 4 16 648 4 16 651 4 16 654 4 |  |  |
| 41 42 43 44 45 | 12 735 4 12 738 3 12 741 3 12 744 3 12 747 3 | 12 524.25 4 12 527.25 3 12 530.25 3 12 533.25 3 12 536.25 3 | 16 927 4 16 930 4 16 933 4 16 936 4 16 939 4 | 16 657 4 16 660 4 16 663 4 16 666 4 16 669 4 |  |  |
| 46 47 48 49 50 | 12 750 3 12 753 3 12 756 3 12 759 3 12 762 4 | 12 539.25 3 12 542.25 3 12 545.25 3 12 548.25 3 12 551.25 4 | 16 942 4 16 945 4 16 948 4 16 951 4 16 954 3 | 16 672 4 16 675 4 16 678 4 16 681 4 16 700.5 3 |  |  |
| 51 52 53 54 55 | 12 765 4 12 768 4 12 771 4 | 12 554.25 4 12 557.25 4 12 560.25 4 | 16 957 3 16 960 3 16 963 3 16 966 3 16 969 3 | 16 703.5 3 16 706.5 3 16 709.5 3 16 712.5 3 16 715.5 3 |  |  |
| 56 57 58 59 60 |  |  | 16 972 3 16 975 3 16 978 3 16 981 3 16 984 3 | 16 718.5 3 16 721.5 3 16 724.5 3 16 727.5 3 16 730.5 3 |  |  |
| 61 62 63 64 65 |  |  | 16 987 3 16 990 4 16 993 4 16 996 3 16 999 3 | 16 733.5 3 16 736.5 4 16 739.5 4 16 742.5 3 16 745.5 3 |  |  |
| 66 67 68 69 70 |  |  | 17 002 3 17 005 3 17 008 3 17 011 3 17 014 3 | 16 748.5 3 16 751.5 3 16 754.5 3 16 757.5 3 16 760.5 3 |  |  |
| 71 72 73 74 75 |  |  | 17 017 3 17 020 3 17 023 3 17 026 3 17 029 3 | 16 763.5 3 16 766.5 3 16 769.5 3 16 772.5 3 16 775.5 3 |  |  |
| 76 77 78 |  |  | 17 032 3 17 035 3 17 038 3 | 16 778.5 3 16 781.5 3 16 784.5 3 |  |  |

Table of frequencies (kHz) assignable to ship and coast stations
 for data transmission (kHz) 1 to be used from 1 January 2017

| Channel No. | 22 MHz |  | 25/26 MHz |  |
| --- | --- | --- | --- | --- |
|  | Coast TX (Ship RX) | Coast RX (Ship TX) | Coast TX (Ship RX) | Coast RX (Ship TX) |
| 1 2 3 4 5 | 22 354 2 22 357 2 22 360 2 22 363 2 22 366 2 | 22 354 2 22 357 2 22 360 2 22 363 2 22 366 2 | 25 142 2 25 145 2 25 148 2 25 151 2 25 154 2 | 25 142 2 25 145 2 25 148 2 25 151 2 25 154 2 |
| 6 7 8 9 10 | 22 369 2 22 372 2 22 378 4 22 381 4 22 391 3 | 22 369 2 22 372 2 22 181.5 4 22 184.5 4 22 187.5 3 | 25 157 2 25 160 2 25 163 2 25 166 2 25 169 2 | 25 157 2 25 160 2 25 163 2 25 166 2 25 169 2 |
| 11 12 13 14 15 | 22 394 3 22 397 3 22 400 3 22 403 3 22 406 3 | 22 190.5 3 22 193.5 3 22 196.5 3 22 199.5 3 22 202.5 3 | 25 172 2 25 175 2 25 178 2 25 181 2 25 184 2 | 25 172 2 25 175 2 25 178 2 25 181 2 25 184 2 |
| 16 17 18 19 20 | 22 409 3 22 412 3 22 415 3 22 418 3 22 421 3 | 22 205.5 3 22 208.5 3 22 211.5 3 22 214.5 3 22 217.5 3 | 25 187 2 25 190 2 25 193 2 25 196 2 25 199 2 | 25 187 2 25 190 2 25 193 2 25 196 2 25 199 2 |
| 21 22 23 24 25 | 22 424 3 22 427 3 22 430 3 22 433 3 22 436 3 | 22 220.5 3 22 223.5 3 22 226.5 3 22 229.5 3 22 232.5 3 | 25 202 2 25 205 2 26 103.5 3 26 106.5 3 26 109.5 3 | 25 202 2 25 205 2 25 123 3 25 126 3 25 129 3 |
| 26 27 28 29 30 | 22 439 3 22 442 3 22 447 4 22 450 4 22 453 3 | 22 235.5 3 22 238.5 3 22 243.5 4 22 246.5 4 22 249.5 3 | 26 112.5 3 26 115.5 3 26 118.5 3 | 25 132 3 25 135 3 25 138 3 |
| 31 32 33 34 35 | 22 456 3 22 459 3 22 462 3 22 465 3 22 468 3 | 22 252.5 3 22 255.5 3 22 258.5 3 22 261.5 3 22 264.5 3 |  |  |
| 36 37 38 39 40 | 22 471 3 22 474 3 22 477 3 22 480 3 22 483 4 | 22 267.5 3 22 270.5 3 22 273.5 3 22 276.5 3 22 279.5 4 |  |  |
| 41 42 43 44 45 | 22 486 4 22 489 4 22 492 4 22 495 4 22 498 4 | 22 282.5 4 22 286 4 22 289 4 22 299.5 4 22 302.5 4 |  |  |
| 46 47 48 49 50 | 22 501 4 22 504 4 22 507 4 22 510 4 22 513 3 | 22 305.5 4 22 308.5 4 22 311.5 4 22 314.5 4 22 317.5 3 |  |  |
| 51 52 53 54 55 | 22 516 3 22 519 3 22 522 3 22 525 3 22 528 3 | 22 320.5 3 22 323.5 3 22 326.5 3 22 329.5 3 22 332.5 3 |  |  |
| 56 57 58 59 60 61 | 22 531 3 22 534 3 22 537 3 22 540 3 22 543 3 22 546 3 | 22 335.5 3 22 338.5 3 22 341.5 3 22 344.5 3 22 347.5 3 22 350.5 3 |  |  |

Reason: Specifies the frequencies for data transmissions by new digital technologies.

ADD	ASP/1.9/22

RESOLUTION [A1.9_NBDP] (WRC-12)

Application and abrogation of certain provisions of the 
Radio Regulations as revised by WRC-12

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that this conference has adopted a revision to the Radio Regulations (RR) in accordance with its terms of reference which will enter into force on [1 January 2013];

b)	that some of the provisions, as amended by this conference, need to apply as of a later date;

c)	that as a general rule, new and revised Resolutions and Recommendations enter into force at the time of signing of the Final Acts of a conference;

d)	that as a general rule, Resolutions and Recommendations which a WRC has decided to suppress are abrogated at the time of the signing of the Final Acts of the conference,

resolves

1	that, as of 1 January 2017, the following provisions of the RR, which are suppressed by this conference, shall be abrogated: “Table of frequencies for two-frequency operation by coast stations (kHz) to be used before 1 January 2017”, Section II of Part B of Appendix 17;

2	that, as of 1 January 2017, the following provisions, as established by this conference, shall enter into force: “Table of frequencies for two-frequency operation by coast stations (kHz) to be used from 1 January 2017”, Section II of Part B of Appendix 17.

Reason: Needs new Resolution to support the modifications of Appendix 17.

MOD	ASP/1.9/23

ARTICLE  59

Entry into force and provisional application
            of the Radio Regulations     (WRC-12)

ADD	ASP/1.9/24

59.A109	–	the revised provisions for which other effective dates of application are stipulated in Resolution [A1.9_NBDP] (WRC-12):

Reasons: Needs new article to support the modifications of Appendix 17.

SUP	ASP/1.9/25

Resolution 351 (Rev.WRC-07)

Review of the frequency and channel arrangements in the HF bands allocated to the maritime mobile service contained in Appendix 17 with a view
to improving efficiency through the use of new digital technology
by the maritime mobile service

Reason: No further studies are needed.

____________

1 	Within the non-shaded boxes.